PURPOSE

This procedure applies to the delivery of bulk gas from mobile tankers into fixed tanks. The site should nominate a member of staff to be responsible for this procedure along with the checking and storing of the gas. The procedure is designed to minimise the risk of escaping gases during delivery through the overfilling of tanks and incorrect delivery procedures.

RESPONSIBILITIES

Key Roles

- Site/Transport Manager/ Supervisor

- Facilities employees

- Security/ Gate person

- Environmental Co-ordinator (where relevant)

Refer to EMS1: Compass Group - Environmental Aspects Register

PROCEDURE

Health and Safety Information

- Fire Safety: SMOKING IS STRICTLY FORBIDDEN BY STAFF INVOLVED WITH OR ADJACENT TO THE TANKER DURING DELIVERY. Ensure that there is a fire extinguisher nearby.

- Staff must be aware of vehicle movements at all times. The tanker must be parked safely in order to minimise obstructions in the yard for other vehicles. Where necessary, traffic cones should be used to re-direct other vehicles away from the tanking point.

Environmental Information

- The source of any escaped gases should be identified and stopped without delay.

- The local environmental co-ordinator must be informed immediately of any significant escape.

PRIOR TO DELIVERY

- Contact suppliers and order the required amount of gas and agree on a suitable delivery date and time.

- After arranging the delivery, ensure that the delivery area is kept clear and that appropriate staff will be available to oversee the delivery.

- Ensure that all tanks/vessels and associated valves and fixings are in a good condition free from any tripping hazards and ready for the delivery.

DURING DELIVERY

- When the delivery tanker arrives on site, direct the driver to the correct parking area and carry out a site induction.

- Request a copy of the purity note/certificate where relevant.

- Prior to tanking, re-check the area around the tank(s) to ensure that there are no tripping hazards and all associated valves and fixings are in a good condition.

- Check that the tank/vessel pressure is not below 10 Bar as this is designated as warm, and MUST only be filled with a competent engineer present. Check the amount of product in the tank and full indicators.

- If all is correct and in order give the driver clearance to connect pipes and tank the delivery.

- Under supervision allow the gas delivery to commence.

THE GAS DELIVERY MUST BE SUPERVISED AT ALL TIMES

- All hoses and connectors must be constantly checked for leakage.

- If a problem occurs during delivery then the tanking must cease immediately.

- Report any incidents to the Local Environmental Co-ordinator immediately.

ONCE THE DELIVERY HAS BEEN TANKED

- Collect and sign for delivery note from the driver and check that the amount is correct.

- Ensure that the tanker driver reinstates any isolated gauges.

- Re-check tank level indicators.

- Complete all necessary paperwork which should be returned to the office for recording and invoicing purposes.

HEALTH & SAFETY AND ENVIRONMENTAL ISSUES

MUST BE CONSIDERED AT ALL TIMES