| #### Ref. IRE 10 | #### Task/Activity: Operation of Juicer |  |  |  | #### Assessment date: |
| --- | --- | --- | --- | --- | --- |
| #### Unit No: | #### Unit name/location: |  |  |  | #### Assessed by: |
| Hazard |  |  | Safety Risk | #### Safe System of Work / Control Measures |  |
| Contact with moving machine part Electrical Hazard Manual Handling Juicer used in this unit is: |  |  | Entanglement Cuts and lacerations Crush injuries Amputation Electric shock Back and muscle strain | - Juicer to be used & cleaned only by trained employees. - Juicer to be maintained by competent persons in accordance with manufacturer’s guidance - Juice to be fitted with all necessary guarding & safety interlocks , which must be in good working order - Juicers should be placed on a stable surface - Any spillages should be cleaned up immediately - Employees must not reach into the machine - Always was fruit & vegetables before processing them - Peek fruits with inedible or thick skins - Stone large- stoned fruit - Check the on/ off button before commencing - Ensure the safety cut offs- handle, cover and peel bucket are in place before operation - Place container underneath the juice outlined and select desired speed - Insert fruit through the feeding tubes and press down with the food pusher - To stop the appliance turn the selector to On/Stand by position - To prevent juice dripping when not extracting, close the anti- drip tap Cleaning - With the appliance switched off, unplug it by removing the plug - Remove the handling lifting it up from the front - Remove the cover lifting it upwards & separate it from the peel bucket - Dismantle the anti- drip tap & coaster - Dismantle the grater- filter & the stainless steel tank assembly, by pulling the bucket upwards, following which they should come apart easily Note: Handle the grater- filter assembly with care- do not touch sharp edges- use suitable hand protection - Place all components in the dishwasher - Clean the outside of the appliance using a damp cloth with suitable detergent, rinse and with a damp cloth - When putting the grater- filter back, make sure it is correctly mounted, pushing it down firmly as far as it will go Ensure the machine is powered off before attempting to remove any blockages. - Implement the Safe System of Work/Control Measures in Electrical Safety Risk Assessment, Ref. MAN 05, including an annual PAT test and the manual handling safety precautions . |  |
| Who may be affected by the task/activity? (Tick all applicable boxes) |  |  |  |  |  |
| Compass employees/Agency staff |  |  |  |  |  |
| Customers / Client staff |  |  |  |  |  |
| Visitors / Members of the Public |  |  |  |  |  |
| Maintenance personnel |  |  |  |  |  |
| Delivery personnel |  |  |  |  |  |
| Other (specify below) |  |  |  |  |  |
| IMPORTANT - This risk assessment should be reviewed every 3 years, or whenever there is a significant change in the task or activity and following any accident or incident involving this task or activity. This risk assessment must be retained for a period of 6 years. |  |  |  |  |  |